# Analog Circuit Design

Laboratory Report

EEM_WS2025/26

Guided by

FH-Prof. DI Dr. Robert Okorn Stefan Dierer, MSc

Prepared by

Muhammad Adeel Akhtar

Muhammad Rabeet Tahir

01 August 2026

Contents

1	Computer Lab	1

1.1	Stochastic Signals	1

1.2	Noise Analysis with PSpice	3

1.2.1	Task Description	3

1.3	Noise Analysis of a single-stage emitter circuit	3

1.3.1	Schematic of a single-stage emitter circuit	3

1.3.2	Noise voltage spectral density	4

1.4	Noise analysis of a single stage inverting and non-inverting operational amplifier circuit.	4

1.4.1	Task Description	4

1.4.2	Schematic of an inverting and non-inverting amplifier circuit	5

5

2	Active band pass filter	7

2.1	Task Description	7

2.2	Schematic of the 2nd order band pass filter	8

2.3	Calculation of filter components	8

2.3.1	Calculated filter components	8

2.4	Simulation Results	9

2.4.1	Transient Analysis with sine wave input	9

2.4.2	Transient Analysis with unit step input	9

2.4.3	Noise Analysis	10

2.4.4	Monte Carlo Analysis	11

2.5	Measurement Results	13

2.5.1	Frequency Response	13

2.5.2	Pass band gain at center frequency	14

2.5.3	Step response	15

2.6	Comparison of Measured and Simulated Results	16

2.7	Discussion of measured output results	16

3	ADC-driver and anti-aliasing filter	17

3.1	Task Description	17

3.2	Schematic of the ADC-driver with anti-aliasing filter	17

3.3	Calculation of filter components	17

3.3.1	Calculated filter components	18

3.4	Simulation Results	18

3.4.1	Transient Analysis	18

3.4.2	Total output noise	18

3.5	Measurement Results	20

3.5.1	Pass band output signals	20

3.5.2	Step Response	21

3.6	Comparison of Measured and Simulated Results	22

3.7	Calculation of SNR and ENOB on the ADC full-scale input	24

3.8	Discussion of Measured Results	24

4	Switched Capacitor Filter	25

4.1	Task Description	25

4.2	Schematic of switched capacitor filter	25

4.3	Measurement Results	25

4.3.1	Magnitude Response	25

4.3.2	Bode plot comparison	27

4.3.3	Input signal near tuning frequency	29

4.3.4	Switching frequency at output signal	30

4.3.5	Total output noise measurement	32

4.4	Calculation of SNR and ENOB	34

4.4.1	Total Input Noise Voltage	34

4.4.2	Calculation of SNR and ENOB	34

4.5	Discussion on Measurements	35

5	Instrument Device List	36

6	Literature	36

7	Date and Signatures	36

8	Appendix	37

8.1	Active band pass filter components calculations	37

8.2	ADC-driver and anti-aliasing filter components calculations	40

List of Figures

Figure 1: Graphical Diagram	1

Figure 2: autocorrelation	2

Figure 3: Spectral Power Density	2

Figure 4: Cross Correlation	3

Figure 5: Schematic of Single Stage Emitter Circuit	3

Figure 6: Voltage Noise Spectral Voltage Density (in red) and Total Output Noise (in blue)	4

Figure 7: Schematic of inverting and non-inverting amplifier	5

Figure 8: Noise Spectral Voltage Density (Non-Inverting Amplifier)	5

Figure 9: Noise Spectral Voltage Density (Inverting Amplifier)	6

Figure 10: Resistance change effect on Noise Spectral Voltage Density (On Inverting Amplifier)	6

Figure 11: Schematic of the second order band pass filter (Butterworth)	8

Figure 12: Sine Wave Transient Analysis	9

Figure 13: Unit Step Response	9

Figure 14: Noise Density in ‘V2/Hz’	10

Figure 15: Total Output Noise Voltage in ‘V’	10

Figure 16: 3dB Bandwidth	11

Figure 17: Center Frequency	12

Figure 18: Gain Variation in dB	12

Figure 19: Pass Band Gain (at center frequency)	14

Figure 20: Step Response of the filter	15

Figure 21: Comparison between Measured and Calculated Responses	16

Figure 22: Schematic of the ADC-driver with anti-aliasing filter.	17

Figure 23: Time domain simulation with sine wave input	18

Figure 24: Noise Power Density (V2/Hz)	19

Figure 25: Integrated differential output-noise voltage for V(Uout+,Uout−), integrated from 1 Hz to 10 MHz.	19

Figure 26: Pass band signals	21

Figure 27: Step-Response of the Filter	22

Figure 28: Differential magnitude and phase responses of the ADC driver with anti-aliasing filter (Wrapped Phase).	23

Figure 29: Differential magnitude and phase responses of the ADC driver with anti-aliasing filter (Unwrapped Phase).	23

Figure 30: Schematic of the switched-capacitor filter with pre-amplifier stage.	25

Figure 31: Magnitude response of SC-filter with 500 kHz and 1 MHz tuning frequencies (Wrapped Phase)	27

Figure 32: Magnitude response of SC-filter with 500 kHz and 1 MHz tuning frequencies (Unwrapped Phase)	28

Figure 33: Input and aliased output signals for an input frequency near the switching frequency.	29

Figure 34: Switching frequency visibility at the output	30

Figure 35: Switching Frequency	31

Figure 36: Measurement of the total output noise of the circuit with the pre-amplifier and SC filter and an additional second stage amplifier. The input signal was connected directly to GND.	32

Figure 37: Total Output Noise with Rg = 51ohm	33

Figure 38: Total Output Noise with Rg = 100kohm	33

Figure 39: Schematic of the first-stage multi-feedback band pass filter.	37

List of Tables

Table 1: Calculated values for Butterworth filter	8

Table 2: Frequency Response Data	13

Table 3: Calculated and implemented resistor and capacitor values for the ADC Driver with anti-aliasing filter.	18

Table 4: Measured frequency response of the ADC-driver and anti-aliasing filter (𝐕𝐂𝐂=+5V, 𝐕𝐄𝐄=0V).	20

Table 5: Measured magnitude response of the SC-filter with 500 kHz tuning frequency (VCC​=+5V, VEE​=−5V).	25

Table 6: Measured magnitude response of the SC-filter with 1 MHz tuning frequency (VCC​=+5V, VEE​=−5V).	26

# Computer Lab

## Stochastic Signals

Value used for given simulations

- Number of Samples = 1000

- Amplitude = 8.2

- f / Hz = 5

- Standard deviation = 5

- Demsion size = 100

Graphic Diagram

Figure 1: Graphical Diagram

Auto-Correlation

Peak at zero lag position (1000, where shifted version of signal exactly matches with original signal) which shows period nature of sine wave as well. For the noise signal, the autocorrelation rapidly decayed to zero for increasing time delay, indicating that the noise signal was random and uncorrelated over time.

Figure 2: autocorrelation

Spectral Power Density

Figure 3: Spectral Power Density

The resulting spectrum showed a distinct peak at the sine wave frequency (5 cycles per 1000 samples) so peak is around 5 and 995 because of symmetry property, representing the deterministic component of the signal. In addition, a continuous wide band noise was observed across the frequency range, caused by the noise component.

Cross Correlation

For the two sine wave signals, the cross-correlation showed a clear peak at a specific time delay, indicating a strong correlation between the signals. For the two noise sources, no significant correlation peak was observed, confirming that the noise signals were uncorrelated and random.

Figure 4: Cross Correlation

As dimension size is 100, cross correlation peak at 100 samples delayed as compared to auto correlation.

## Noise Analysis with PSpice

### Task Description

Noise analysis is the key aspect in designing and analyzing op-amp circuits to get familiar with non-ideal output. It ensures circuits performance, stability, and functionality in real-time circuits where signals are distributed by external noise sources. Noise analysis is used in evaluation of unwanted electrical signals that disturb the operation of electrical circuits. Noise analysis deals with the fluctuation in the voltages or current flowing through the operational amplifier because of noise sources. OrCAD PSpice is used to evaluate and verify the circuit’s noise voltage spectral density in this task.

## Noise Analysis of a single-stage emitter circuit

### Schematic of a single-stage emitter circuit

In this part single stage emitter circuit is designed using BC547C bipolar junction transistor as an amplifier circuit.

Figure 5: Schematic of Single Stage Emitter Circuit

Figure 6: Voltage Noise Spectral Voltage Density (in red) and Total Output Noise (in blue)

Noise voltage spectral density

Noise spectral voltage density V(ONOISE) in V/√Hz and SQRT(S(NTOT(ONOISE)) is Integrated Total RMS Noise in V.

- At low frequencies R5 and C1 function as high pass filter so noise density is low

- At mid band range, noise is flat (white noise) mostly due to thermal noise of Resistors.

- At high frequencies noise density drops because of amplifier bandwidth is limited.

## Noise analysis of a single stage inverting and non-inverting operational amplifier circuit.

### Task Description

In this task, an inverting and non-inverting amplifier is designed and simulated on PSpice to see the noise spectral voltage density of the circuit. Figure 7 displays the schematic of amplifier’s circuit and figures 8, 9 and 10 show the noise voltage spectral density (non-inverting), noise voltage spectral density (inverting), and resistance change effect on inverting amplifier respectively.

### Schematic of an inverting and non-inverting amplifier circuit

Figure 7: Schematic of inverting and non-inverting amplifier

Figure 8: Noise Spectral Voltage Density (Non-Inverting Amplifier)

Figure 9: Noise Spectral Voltage Density (Inverting Amplifier)

Both results look similar because resistor values are equal to achieve almost same gain.

Figure 10: Resistance change effect on Noise Spectral Voltage Density (On Inverting Amplifier)

R8 changed from 10K to 5k so noise density also decreased.

Noise Spectral Voltage Density for inverting and non-inverting amplifier is shown above.

Noninverting Amplifier Noise Spectral Voltage Density (V/√Hz )

- At low frequencies noise density is high because of flicker noise

- From range 100Hz to 500kHz flat noise density because of thermal noise

- At Higher frequencies noise density rolls off is due to limited gain bandwidth of AD823an Opamp.

- GBW=Acl×fBW=11×1.31×106=14.4MHz (fBW calculated from bode plot in simulation)

- Rolls off start after 14.4Mhz onwards as we move to higher frequencies.

# Active band pass filter

## Task Description

In this task a second order active band pass filter with the dual Op-Amp AD823an/AD is designed, by cascading two staggered tuned first order band pass filters by using the multi feedback topology (MFB) to build the two filter stages. Filter has following specifications

- Passband: 4000 Hz - 5000 Hz

- Gain: 20 dB

- Response Type: Butterworth

The design process began with the calculation of key parameters, including the damping factor (α). In order to calculate the components of filters the equation of α is solved by using Wolfram Alpha and then values of capacitors and resistors are calculated by using filter equations.

The circuit is simulated in OrCAD PSpice, and the following analyses are conducted:

- Transient analysis using unit-step and sine wave inputs

- AC analysis to generate Bode plots

- Noise analysis (noise density, total output noise voltage)

- Monte Carlo analysis with component tolerances histograms (for 3dB bandwidth, center frequency, gain variation).

Next step is implementation on the breadboard, following steps are performed.

- Assemble circuit using Op-Amp AD823an/AD with supply voltages of +5V and-5V

- Measure frequency response using oscilloscope’s ratio and phase measurement function.

- Measure pass band gain (at center frequency)

- Measure 3dB corner frequencies

- Measure step-response of the filter

Final step is to compare the simulated and measured values using Bode Diagram

## Schematic of the 2nd order band pass filter

Figure 11: Schematic of the second order band pass filter (Butterworth)

## Calculation of filter components

To calculate value of components we need transfer function of above filter, complete calculations and derivations are written in Appendix 8.1.

### Calculated filter components

Table 1: Calculated values for Butterworth filter

| Component | Calculated Ω | Used Ω |
| --- | --- | --- |
| R1 | 5440 | 5600 |
| R2 | 48820 | 51000 |
| R3 | 321 | 300 |
| R4 | 4650 | 4700 |
| R5 | 41790 | 43000 |
| R6 | 274 | 270 |
| C1 = C2 = C3 = C4 | 10nF |  |

## Simulation Results

In this section, transient analysis of a second order active band pass filter with the dual Op-Amp AD823an/AD, by cascading two staggered tuned first order band pass filters by using the multi feedback topology is done with sine wave and unit step input.

### Transient Analysis with sine wave input

VIN(P) = 306 mV, VOUT(P) = 3.5175 V, Freq = 4.5kHz, Gain = 21.21dB

Figure 12: Sine Wave Transient Analysis

### Transient Analysis with unit step input

VIN = 1V, as step contains all frequencies, band pass only allows certain range of frequencies.

Figure 13: Unit Step Response

### Noise Analysis

#### Noise density

Spectral power density of noise G(f) is maximum around 4.5499kHz (center frequency)

G(f)max = 15.696p V2/Hz

Figure 14: Noise Density in ‘V2/Hz’

#### Total output noise voltage

Total Output noise voltage (integrated noise over frequency) of this filter is

Vnoise (RMS) = 129.940uV		(At Nominal Temperature (Pspice) TNOM = 27.0°C)

Figure 15: Total Output Noise Voltage in ‘V’

### Monte Carlo Analysis

For all given histogram Analysis following values are used.

- Number of runs = 100

- Distribution = Uniform

- Random Seed Number = 2048

- Tolerance of Components = 1%

#### Monte Carlo analysis with histogram for 3 dB bandwidth

According to design mean is at 0.9kHz bandwidth.

Figure 16: 3dB Bandwidth

#### Center frequency

Mean is at 4.485kHz which is our expected center frequency.

Figure 17: Center Frequency

#### Gain Variation

Mean of samples is at 20.95dB and deviations are because of tolerance in components.

Figure 18: Gain Variation in dB

## Measurement Results

### Frequency Response

The Butterworth filter's frequency response, including magnitude and phase, is measured by supplying sine wave to the circuit with the help of function generator and using the oscilloscope’s ratio and phase functions covering frequencies from 3.4 kHz to 6 kHz, as shown in Table 2.

Table 2: Frequency Response Data

| Frequency in Hz | Magnitude in dB | Phase in degree |
| --- | --- | --- |
| f/Hz | G/ dB | Φ/ ˚ |
| 3400 | -0.1 | 151 |
| 3500 | 1.4 | 149.8 |
| 3600 | 3.1 | 145.3 |
| 3700 | 4.9 | 142.1 |
| 3800 | 6.8 | 135.5 |
| 3900 | 8.8 | 128.2 |
| 4000 | 11.0 | 120.5 |
| 4100 | 13.3 | 110.6 |
| 4270(fL) | 16.9 | 81.3 |
| 4300 | 17.4 | 76.7 |
| 4400 | 18.7 | 57.6 |
| 4500 | 19.5 | 37.5 |
| 4700(fc) | 19.9 | -1.09 |
| 4750 | 19.9 | -10 |
| 4800 | 19.8 | -18.3 |
| 4900 | 19.5 | -34.5 |
| 5000 | 18.9 | -50.3 |
| 5100 | 17.9 | -66.7 |
| 5190(fH) | 16.9 | -78.4 |
| 5300 | 15.5 | -92.5 |
| 5400 | 14.1 | -102.8 |
| 5500 | 12.6 | -110.4 |
| 5600 | 11.2 | -117 |
| 5700 | 9.9 | -122.8 |
| 5800 | 8.6 | -127.3 |
| 5900 | 7.4 | -133.2 |
| 6000 | 6.2 | -135.3 |

### Pass band gain at center frequency

- Gain at center frequency, fc = 4.7kHz

- Channel 1(in yellow) is Output Voltage (2.85Vp)

- Channel 2(in green) is Input Voltage (306mVp)

Figure 19: Pass Band Gain (at center frequency)

### Step response

- Channel 1(in green) is Input Voltage

- Channel 2(in yellow) is Output Voltage

Figure 20: Step Response of the filter

## Comparison of Measured and Simulated Results

Measurement results are close to simulation results; deviation occurs because of tolerance in components and effect of parasitic (inductance of jumping wires.)

Figure 21: Comparison between Measured and Calculated Responses

## Discussion of measured output results

In this task, a second-order bandpass filter was designed using the OpAmp AD823an/AD. The design was based on the properties of a Butterworth filter, and a multi-feedback architecture was implemented with the OpAmp. The filter’s characteristics were simulated in the PSpice environment. Following the simulation, the circuit was constructed on a breadboard, and the filter’s behavior was measured. An overlap was observed between the simulated and measured values. The small shifts in the center frequency and the 3 dB cutoff frequencies are reasonable and can be explained by the tolerances of the components.

Summary of Measured Results:

- Center frequency: 4.7 kHz with 19.9 dB gain

- Lower 3dB cutoff frequency: 4.27 kHz with 16.9 dB gain

- Upper 3dB cutoff frequency: 5.19 kHz with 16.9 dB gain

How could the output noise be measured?

To measure the output noise of the Butterworth filter, the input can be grounded, and a high-gain amplifier circuit can be connected to the output. This amplifier will amplify the noise, making it visible and measurable. The noise can then be observed and quantified using RMS (Root Mean Square) function on an oscilloscope.

# ADC-driver and anti-aliasing filter

## Task Description

In the task, a single single-ended to differential driver is designed with the help of second order active low pass filter for the AD7626 ADC by using the AD823an/AD dual Op-Amp. The cutoff for the lowpass should be 40kHz and additionally a high pass with a cutoff frequency of 100Hz had to be added. The total differential gain is supposed to be 20 and a DC offset must be provided for the ADC signal (2.5V on both differential outputs).

## Schematic of the ADC-driver with anti-aliasing filter

Figure 22: Schematic of the ADC-driver with anti-aliasing filter.

## Calculation of filter components

In this circuit, C 1 and C 2 were predefined as part of the design specifications. The filter was implemented using a Multiple-Feedback (MFB) topology for a low-pass response. A detailed description of the general formulas and calculations for the derivation of the component values can be seen in appendix 8.2.

### Calculated filter components

All resistors’ values are in ohm(Ω) and capacitors in farad(F).

Table 3: Calculated and implemented resistor and capacitor values for the ADC Driver with anti-aliasing filter.

| Component | Calculated value | Implemented value |
| --- | --- | --- |
| R1 | 852 | 910 |
| R2 | 8519 | 9100 |
| R3 | 2635 | 2700 |
| R4 | 1000 | 1000 |
| R5 | 1000 | 1000 |
| C1 | 150p | 150p |
| C2 | 4.7n | 4.7n |
| C3 | 1.85u | 2u |
| C4 | 100n | 100n |
| C5 | 10u | 10u |

## Simulation Results

### Transient Analysis

Both outputs are (180°) out of phase with equal magnitude.

- Uout+ is output of first Opamp (inverts input with gain of -10) in blue

- Uout- is output of second Opamp (inverts Uout+ with gain of 1) in red

- Uin = 100mVp, Freq = 10k

Figure 23: Time domain simulation with sine wave input

### Total output noise

Figure 24: Noise Power Density (V2/Hz)

The noise-analysis output variable is defined as V(Uout+,Uout-). Therefore, the plotted integrated noise represents the RMS differential noise voltage directly across the ADC inputs, rather than the noise at only one output.

Figure 25: Integrated differential output-noise voltage for V(Uout+,Uout−), integrated from 1 Hz to 10 MHz.

## Measurement Results

Table 4: Measured frequency response of the ADC-driver and anti-aliasing filter (𝐕𝐂𝐂=+5V, 𝐕𝐄𝐄=0V).

| Frequency in Hz | Magnitude in dB | Phase in degree | Unwrapped Phase |
| --- | --- | --- | --- |
| f/Hz | G/ dB | Φ/ ˚ | Φ/ ˚ |
| 50 | 17.3 | -118.7 | -118.7 |
| 100 | 21.3 | -136.8 | -136.8 |
| 200 | 23.4 | -155.2 | -155.2 |
| 500 | 24.2 | -168.3 | -168.3 |
| 1000 | 24.4 | -175.4 | -175.4 |
| 2000 | 24.5 | 178.6 | -181.4 |
| 5000 | 24.7 | 170.4 | -189.6 |
| 10000 | 24.7 | 159.8 | -200.2 |
| 20000 | 24.7 | 136.0 | -224.0 |
| 30000 | 23.9 | 108.1 | -251.9 |
| 40000 | 22.0 | 83.8 | -276.2 |
| 60000 | 16.6 | 52.7 | -307.3 |
| 80000 | 11.9 | 34.6 | -325.4 |
| 100000 | 8.0 | 23.8 | -336.2 |

### Pass band output signals

This measurement is performed under following conditions

- Result taken at Uin = 209mVpp, Freq = 1kHz

- Channel 1 (in yellow) is Uout+

- Channel 2 (in green) is Uout-

- Channel 3 (in blue) is Uin

- Math trace M1 (in pink) represents  (Uout+-Uout-).

- Both single-ended outputs are centered at approximately 2.5 V, which provides the required ADC common-mode voltage for single-supply operation.

- Gain =24.4dB

Figure 26: Pass band signals

### Step Response

This measurement is performed under following conditions

- VIN(P) = 200mV (in blue)

- Uout- (in green)

- Uout+ (in yellow)

Figure 27: Step-Response of the Filter

## Comparison of Measured and Simulated Results

The measured gain response closely follows the simulated behavior, with only a slight deviation in values at the lower frequency range. The transient simulation produced the differential gain of approximately (26.02 dB), whereas the maximum measured pass-band gain was 24.7 dB.

In the phase response, fluctuations were observed at certain points, primarily due to variations in oscilloscope readings.

Figure 28: Differential magnitude and phase responses of the ADC driver with anti-aliasing filter (Wrapped Phase).

The oscilloscope gives phase values within the principal range from -180∘to +180∘.The actual phase became more negative than -180∘, the displayed value wrapped to a positive angle. To obtain a continuous phase response for comparison with the PSpice simulation, 360∘is subtracted from the wrapped positive phase values.

Figure 29: Differential magnitude and phase responses of the ADC driver with anti-aliasing filter (Unwrapped Phase).

## Calculation of SNR and ENOB on the ADC full-scale input

The total noise for calculating the SNR was found by performing the PSpice simulation which is 165.411µV(approx.) (from fig25). The output noise was integrated up to 10 MHz to represent the selected system bandwidth used for the SNR calculation.

VFS,PP is twice of 8.192V because of (+- 4.096V)

Vsig,rms= V(FS,pp)22= 8.1922.828=2.896V

SNRdB=20log⁡10Vsig,rmsVn,rms=  20log⁡102.896165.411*10-6= 84.87dB

ENOB=SNRdB-1.766.02=84.87-1.766.02=13.80 bits

## Discussion of Measured Results

To prevent aliasing, we used anti-aliasing filters at the ADC input, in accordance with Nyquist’s sampling theorem. These filters ensured that the input signal frequency remained below half the sample rate. Consequently, unnecessary sampling data was avoided. The measured results closely matched the calculated values.

# Switched Capacitor Filter

## Task Description

The 8th-order switched capacitor (SC) filter was designed, tested, and analyzed using the MAX295 integrated device, paired with a non-inverting pre-amplifier stage built with the AD823an/AD operational amplifier. The circuit was configured with the following specifications:

- Implement a clock-tunable SC filter.

- Integrate a pre-amplifier stage with a gain of 100 before the SC filter.

- Supply Voltage: ±5V dual supply.

- Tuning Frequencies: 1 MHz and 500 kHz.

## Schematic of switched capacitor filter

Figure 30: Schematic of the switched-capacitor filter with pre-amplifier stage.

## Measurement Results

### Magnitude Response

Table 5: Measured magnitude response of the SC-filter with 500 kHz tuning frequency (VCC​=+5V, VEE​=−5V).

| Frequency in Hz | Magnitude in dB | Phase in degree | Phase (Unwrapped) |
| --- | --- | --- | --- |
| f/Hz | G/ dB | Φ/ ˚ | Φ/ ˚ |
| 4000 | 39.2 | -122.3 | -122.3 |
| 5000 | 39.1 | -155.2 | -155.2 |
| 6000 | 39.2 | 170.6 | -189.4 |
| 7000 | 39.2 | 135.1 | -224.9 |
| 8000 | 39.0 | 93.8 | -266.2 |
| 9000 | 38.4 | 49.2 | -310.8 |
| 10000 | 36.6 | -2.4 | -362.4 |
| 11000 | 32.8 | -54.6 | -414.6 |
| 12000 | 27.6 | -92.5 | -452.5 |
| 13000 | 22.2 | -125.1 | -485.1 |
| 14000 | 17.00 | -149.4 | -509.4 |
| 15000 | 12.1 | -170.4 | -530.4 |
| 16000 | 7.4 | 178.8 | -541.2 |
| 17000 | 2.9 | 162.5 | -557.5 |
| 17300 | 1.6 | 152.4 | -567.6 |
| 17500 | 0.8 | 151.0 | -569.0 |

Note: 1) DC-offset = 0 V, Vin = 50 mVpp

Table 6: Measured magnitude response of the SC-filter with 1 MHz tuning frequency (VCC​=+5V, VEE​=−5V).

| Frequency in Hz | Magnitude in dB | Phase in degree | Phase (Unwrapped) |
| --- | --- | --- | --- |
| f/Hz | G/ dB | Φ/ ˚ | Φ/ ˚ |
| 4000 | 39.3 | -62.9 | -62.9 |
| 5000 | 39.3 | -77.8 | -77.8 |
| 6000 | 39.3 | -94.5 | -94.5 |
| 7000 | 39.3 | -110.7 | -110.7 |
| 8000 | 39.3 | -125.6 | -125.6 |
| 9000 | 39.3 | -143.6 | -143.6 |
| 10000 | 39.3 | -159.8 | -159.8 |
| 11000 | 39.3 | -176.4 | -176.4 |
| 15000 | 39.1 | 108.9 | -251.1 |
| 20000 | 36.4 | -9.5 | -369.5 |
| 21000 | 34.8 | -33.9 | -393.9 |
| 22000 | 32.7 | -60.1 | -420.1 |
| 24000 | 27.5 | -102.4 | -462.4 |
| 25000 | 24.8 | -118.9 | -478.9 |
| 26000 | 22.0 | -132.5 | -492.5 |
| 27000 | 19.4 | -145.7 | -505.7 |
| 30000 | 11.8 | -178.5 | -538.5 |
| 32000 | 7.1 | 164.4 | -555.6 |
| 35000 | 0.4 | 144.2 | -575.8 |

Note: 1) DC-offset = 0 V, Vin = 50 mVpp

### Bode plot comparison

Figure 31 illustrates the measured magnitude response of a switched-capacitor (SC) filter at two different tuning frequencies, 500 kHz and 1 MHz. At lower frequencies, both configurations maintain a stable gain of approximately 39.3 dB. As the frequency increases, the gain remains constant until reaching the cutoff region, where it starts to decline sharply.

A key observation is that the cutoff frequency shifts higher with the increased tuning frequency. The 500 kHz tuning results in an earlier roll-off, meaning it attenuates signals at a lower frequency compared to the 1 MHz tuning, which extends the passband before attenuation begins. This confirms that the SC filter’s cutoff frequency is directly proportional (ratio = fclk⁄fc = 50 ) to the tuning frequency.

Both configurations exhibit a steep roll-off beyond their respective cutoff points, characteristic of high-order SC filters. The higher tuning frequency (1 MHz) allows for a wider passband, making it suitable for applications requiring higher-frequency signal processing. The lower tuning frequency (500 kHz) is effective for narrower bandwidth applications, providing stronger attenuation beyond the cutoff.

Overall, the results validate the frequency-dependent nature of switched-capacitor filters and demonstrate their tunability through clock frequency adjustments.

Shift in phase response is because of wrapping effect, oscilloscope jumps phase from -180° to +180. Therefore, the measured phase values were unwrapped in MATLAB by subtracting 360∘after each phase discontinuity.

ϕadjusted =  ϕmeasured - 360°

Figure 31: Magnitude response of SC-filter with 500 kHz and 1 MHz tuning frequencies (Wrapped Phase)

Figure 32: Magnitude response of SC-filter with 500 kHz and 1 MHz tuning frequencies (Unwrapped Phase)

### Input signal near tuning frequency

Aliasing occurs when input signal frequency approaches the sampling frequency, as demonstrated in given figure.

Figure 33: Input and aliased output signals for an input frequency near the switching frequency.

Waveform Description:

Yellow Trace (Channel 1): Represents the signal near 500kHz tuning frequency.

Green Trace (Channel 2): Shows the aliased output sine wave visible on the output of the SC filter.

### Switching frequency at output signal

The presence of periodic high-frequency spikes in the output signal (green trace), synchronized with the 500 kHz clock signal, indicates the occurrence of clock crosstalk.

Figure 34: Switching frequency visibility at the output

Traces:

Yellow Trace (Channel 1): Displays the input signal 10kHz.

Green Trace (Channel 2): Shows the output signal.

Figure 35: Switching Frequency

- Time base: 2 µs/div

- Cursor spacing: ΔX = 2.000 µs

Therefore:

1ΔX=500 kHz

which matches the 500 kHz switching clock.

### Total output noise measurement

An additional low-pass filter and inverting amplifier were connected to the SC-filter circuit's output to measure the total output noise voltage.

Figure 36: Measurement of the total output noise of the circuit with the pre-amplifier and SC filter and an additional second stage amplifier. The input signal was connected directly to GND.

Figure 37: Total Output Noise with Rg = 51ohm

Figure 38: Total Output Noise with Rg = 100kohm

## Calculation of SNR and ENOB

### Total Input Noise Voltage

For Rg = 100kΩ, SC output noise voltage:

Ameas corresponds to two additional noise-measurement amplifier stages

Un,SC-out = Un-meas (Rg = 100kΩ)Ameas

= 1.24151000

= 1.2415 mV

Noise referred to the input before the preamplifier:

Un-in(Rg = 100kΩ) = Un,SC-out (Rg = 100kΩ)Apre (preamplifier gain= 101)

= 1.2415101

= 12.29 µV RMS

Similarly, for Rg = 51Ω, SC output noise voltage

Un,SC-out ==203.2m1000=203.2µV

Noise referred to the input before the preamplifier:

Un-in=203.2µ101=2.02µV RMS

### Calculation of SNR and ENOB

Noise voltage for Rg = 100kΩ

Un,RMS=1.2415 V1000=1.2415 mV

The corresponding mean-square noise voltage is:

Un2=1.2415×10-32=1.5413×10-6 V2

Noise voltage for Rg = 51Ω

Un,RMS=203.2 μV

The corresponding mean-square noise voltage is:

Un2=203.2×10-62=4.1290×10-8 V2

Signal voltage for full scale sine wave

For this calculation, 5 Vppwas selected as the datasheet-based reference full-scale sine-wave level because the MAX295 harmonic-distortion characteristics are specified using a 5 Vppsine-wave input.

VFS,pp=5V

For a sinusoidal signal:

Vs,RMS=Vs,pp22=522=1.768 V

The mean-square signal voltage is:

Vs2=1.7682=3.126 V2
With Rg = 100kΩ

SNR

SNRdB=10log10Vs2Vn2
SNRdB=10log103.1251.5413×10-6

SNRdB=63.07dB

ENOB

ENOB=SNRdB-1.766.02

ENOB=63.07-1.766.02=10.18 bits

With Rg = 51Ω

SNRdB=10log103.1254.1290×10-8
SNR=78.79 dB

The equivalent ENOB is:

ENOB=78.79-1.766.02

ENOB=12.80 bits

## Discussion on Measurements

Aliasing effects occur near the tuning frequency but can be avoided by sampling at a frequency greater than half the signal bandwidth. Additionally, the output noise is relatively high in this case.

# Instrument Device List

Hardware:

- TTi PL303QMD QUAD-MODE DUAL POWER SUPPLY (9949263)

- KEYSIGHT 34450A 5-1/2 Digit Multimeter (9949726)

- KEYSIGHT 33500B Series TRUEFORM WAVERFORM GENERATOR (9949715)

- KEYSIGHT InfiniiVision DSOX3014T (9949705)

- Agilent U1233A True RMS Multimeter (EL03)

Software:

- OrCAD Pspice 25.1

- MATLAB R2025b

- Wolfram Alpha

# Literature

- R. Okorn, Analog Circuit Design Lecture Script V3.13, Graz, 2024.

- R. Okorn, Analog Signal Processing Course Script, Graz, 2021.

- S. Dierer, Noise Analysis with Pspice Handout Part II for OrCAD Capture, Graz, 2024.

- R. Mancini, Op Amps for Everyone, 2002.

- U. Tietze and C. Schenk, Electronic Circuits, Springer Verlag, 2008.

- Maxim Integrated Products, 8th-Order, Lowpass, Switched-Capacitor Filters, Sunnyvale, 1996.

- Texas Instruments, AD823an/AD Dual High-Performance, High-Fidelity Audio Operational Amplifier, Dallas, 2022.

# Date and Signatures

01.08.2026

……………..…..

Date

#### Adeel Akhtar

……………………....…..

Name

Rabeet Tahir

……………………....…..

Name

# Appendix

## Active band pass filter components calculations

Figure 39: Schematic of the first-stage multi-feedback band pass filter.

Transfer Function

Applying KCL at the first node

V1 - VxR1 = Vx - V01sC1 + Vx - 01sC2 + Vx - 0R3

V1 - VxR1 = Vx - V01sC1 + Vx1sC2 + VxR3         (1)

At node 2:

Vx - 01sC2 = 0 - V0R2                                      (2)

sC2Vx = -V0R2

Vx = -V0sR2C2                                                   (3)

Replacing the value of Vx/(1/sC₂) in equation (1) from equation (2).

V1 - VxR1 = Vx - V0sC1 - V0R2 + VxR3    (4)

Putting the value of Vₓ from equation (3) into equation (4):

V1 - Vx = Vx - V0sC1R1 - V0R1R2 + VxR1R3

V1 = Vx - V0sC1R1 - V0R1R2 + VxR1R3 + Vx

V1 = -V0sC1R1R3 + s2R2C2C1R1R3 + R1R3C2s + R1 + R3sR2C2R3

Transfer function H(s) = V₀ᵤₜ / Vᵢₙ :

V0V1 = -sR2C2R3s2R2C2C1R1R3 + s(C1R1R3 + R1R3C2) + R1 + R3

•  Low Pass to Band Pass and Comparison of Coefficients

A(s) = A01 + a1s + b1s2

For converting a low pass filter into a band pass filter, we replace s with:

s = 1Δωns + 1s

After substitution and expansion (shown step by step):

A(s) = A0Δωn2s2b1 + s2Δωn2 + 2b1 + Δωn3sa1 + a1Δωns + b1s4

Dividing by b₁:

A(s) = A0Δωn2s2b11 + a1Δωnb1s + Δωn2 + 2b1b1s2 + Δωna1b1s3 + s4

•  Calculation of α

H(P) = P2H0Δωn2b11 + a1b1ΔωnP + 2 + Δωn2b1P2 + a1b1ΔωnP3 + P4

To implement this filter with 2 cascaded first-order band-pass filters, divide the transfer function:

H(P) = P2H0Δωn2b11 + αPQi + αP21 + 1QiPα + Pα2

Where:

α = Coefficient of comparison

Qᵢ = Quality factor

Solving and comparing coefficients gives:

αQi + 1Qiα = a1b1 ·Δωn

2 + Δωn2b1 = 1α2 + 1Qi2 + α2

With Butterworth coefficients a₁ = 1.4142, b₁ = 1, solving gives the α equation:

α8 - 0.0499α6 - 1.999α4 - 0.0499α2 + 1 = 0

Solved using MATLAB, two values of α:

α1 = 0.9239,  α1 = 1.082

•  Calculation of Resistors

Using α₁ = 1.082. Centre frequency:

fₘᵢₙ = 4000 Hz,  fₘₐₓ = 5000 Hz

fo = fm = fₘᵢₙ · fₘₐₓ = 4000 · 5000 = 4472.1 Hz

Normalized Bandwidth:

Δωn = fₘₐₓ - fₘᵢₙfo = 5000 - 40004472.1 = 0.2236

fₘ1 = fmα = 4472.1361.082 = 4137.036 Hz

fₘ2 = fm · α = (4472.136)(1.082) = 4834.38 Hz

Q = fmB = 4472.1361000 = 4.472

Qi = 4.472 1 + 1.0822 (1)(1.082)(1.4142) = 6.3438

Aₘi = QiQAmb1 = 6.34384.472101 = 4.485

Filter 1:

R2 = Qiπ fₘ₁ C = 6.3438(3.1416)(4137.036)(10 nF) = 48.82 kΩ

R1 = R2-2Aₘᵢ = 48.82 × 10³-2 × -4.485 = 5.44 kΩ

R3 = -Aₘᵢ R₁2Qi2+ Aₘᵢ = (4.485)(5.44 × 10³)26.3432- 4.485 = 321.11 Ω

Filter 2:

R5 = Qiπ fₘ₂ C = 6.3438(3.1416)(4834.38)(10 nF) = 41.79 kΩ

R4 = R5-2Aₘᵢ = 41.79 × 10³-2 × -4.485 = 4.65 kΩ

R6 = -Aₘᵢ R₄2Qi2 + Aₘᵢ = (4.485)(4.65 × 10³)26.3432- 4.485 = 274.48 Ω

## ADC-driver and anti-aliasing filter components calculations

Figure 46. Schematic for 2nd order multi-feedback low pass filter.

At node Vₓ

V4 - VxR1 = Vx - 0R3 + Vx - 01sωC2 + Vx - UoutR2

V4 - VxR1 = VxR3 + sωC2Vx + Vx - VoutR2

At node 0

Vx - 0R3 = 0 - Uout1sωcC1

VxR3 = -sωcC1Uout

Vx = -sωcC1R3Uout  (2)

Replacing the value of Vₓ in equation (1):

V4 + sωcC1R3UoutR1 = -sωcC1R3UoutR3 + sωcC2-sωcC1R3Uout + -sωcC1R3Uout - UoutR2

V4 + sωcC1R3UoutR1 = -sωcC1Uout - s2ωc2C1C2R3Uout - sωcC1R3Uout + UoutR2

V4 + sωcC1R3Uout = -sωcC1R1Uout - s2ωc2C1C2R1R3Uout - sωcC1R3R1Uout + UoutR1R2

V4 = -sωcC1R3Uout - sωcC1R1Uout - s2ωc2C1C2R1R3Uout - sωcC1R3R1Uout + UoutR1R2

V4 = -sωcC1R3Uout - sωcC1R1Uout - s2ωc2C1C2R3R1Uout - sωcC1R1R3Uout + UoutR1R2

V4 = -UoutsωcC1R3 + R1 + R1R3R2 + C1C2R1R3ωc2s2 + R1R2

UoutV4 = -1C1C2R1R3 s²ωc2 + R3 + R1 + R1R3R2 sωcC1 + R1R2

Dividing numerator and denominator by R₁/R₂:

UoutV4 = -R2R1C1C2R3R2ωc2s2 + sωcC1R3 + R2 + R3R2R1 + 1

Comparing with the standard form A(s) = A₀ / (1 + a₁s + b₁s²):

A0 = -R2R1

a1 = ωCC1R3 + R2 + R3R2R1           Eq(a1)

b1 = ωc2C1C2R3R2

From the above equations:

R1 = -R2A0

R3 = b1ωc2C1C2R2

Putting R₁ and R₃ in the ‘a1’ equation and solving the quadratic for R₂:

ωc2C1C2R22 - a1ωcC2R2 + (1 - A0)b1 = 0

Using the Quadratic Formula:

R2 = a1C2 ± a12C22 - 4b1C1C21 - A04π fcC1C2

R1 = -R2A0

R3 = b14π²fc2C1C2R2

Values calculated using MATLAB:

R1 = 851.98 Ω

R2 = 8.519 kΩ

R3 = 2.635 kΩ

R4 = R5

Since:

A0 = -R5R4 = -1